MCP‑EDA Service — Narrative API Overview

————————————————————

What the platform exposes

The flow is decomposed into seven small, single‑purpose HTTP services. Each one lives next to a tool script (Synopsys DC or Cadence Innovus) and listens on its own port. You interact with them over simple POST requests that carry a tiny JSON body.

| Stage | Service Script(s) | Port(s) |
| --- | --- | --- |
| Synthesis | setup_server.py, compile_server.py | 3333, 3334 |
| Physical implementation | floorplan_server.py, powerplan_server.py, placement_server.py, cts_server.py, route_server.py, save_server.py | 3335 – 3340 |

————————————————————

1 · Common request ideas

design – logical design name (e.g. des). Always required.

tech – technology directory under scripts/; defaults to FreePDK45.

force – true forces the stage to rerun even if artefacts already exist.

Index triple g_idx, p_idx, c_idx – pick rows out of the CSVs that hold implementation knobs. Omit them and they silently fall back to 0.

All services accept the three fields above. Each stage adds one or two extra knobs of its own (see below).

————————————————————

2 · What each endpoint does & extra fields

/setup/run (3333) – “DC setup”

Kicks off Synopsys Design Compiler, reads libraries, constrains clocks and performs initial lint.

Additional field: version_idx → row in config/synthesis.csv.

Result: "status":"ok" plus a setup_summary.rpt string and a log_path to the terminal log.

/compile/run (3334) – “DC compile”

Turns RTL into a mapped gate‑level netlist and spits out timing / area / power reports.

No new parameters beyond the common three, but it automatically injects a syn_ver string (you can override it manually if you have multiple synthesis passes).

/floorplan/run (3335) – “Innovus floorplan”

Imports the mapped netlist, draws the die outline, macro rows and I/O rings.

Needs the syn_ver coming from the compile stage.

Reads g_idx + p_idx rows to know the top‑level utilisation and I/O pad rules.

Invents the implementation version name <syn_ver>__g<g_idx>_p<p_idx> reused by later steps.

/power/run (3336)

Builds a basic power grid (stripes, rings, vias). Takes only impl_ver and the usual common fields.

/place/run (3337)

Standard‑cell placement and pre‑CTS optimisation.

Needs the same g_idx, p_idx again to fetch the placement CSV row.

/cts/run (3338)

Clock‑tree synthesis and post‑CTS optimisation.

Adds c_idx to pull a row out of config/cts.csv.

/route/run (3339)

Global + detailed routing, then timing / DRC clean‑up.

Accepts every index (g_idx, p_idx, c_idx) plus an optional top_module override.

Returns plain‑text reports: congestion, DRC, routed timing, etc.

/save/run (3340)

Final export; produces {impl_ver}.gds / .def / .sdf.

No extra knobs; just point it at the implementation you want to package.

————————————————————

3 · Response contract (all services)

Every endpoint returns the same envelope:

status  – "ok" or "error: …"
log_path – absolute path to the raw tool log
reports  – dictionary of any textual reports (inflated on the fly for .gz)

HTTP status code is always 200; CI pipelines rely on the JSON status field instead of network exceptions.

————————————————————

4 · Typical end‑to‑end run

```bash
./run_pipeline.sh \
  --design des \
  --tech FreePDK45 \
  --g 0 --p 0 --c 0
```

The helper script posts to each service in order and threads the generated implementation version between stages. If you prefer you may invoke each /…/run endpoint directly – the JSON snippets above are exactly what the servers need.

————————————————————

5 · When things go wrong

“directory not found” – stale impl_ver; rerun earlier stage or pass "force":true.

Tool licence / crash – read the file pointed to by log_path; service still returns status:"error: …".

Malformed JSON – FastAPI automatically responds with 400.

————————————————————

Contact

Author  Andy Lu – yl996@duke.edu

Repo    https://github.com/AndyLu666/MCP-EDA-Server

Open issues or PRs for new technology nodes, extra reporting hooks, or parameter ideas. Happy hacking!